Creative North Solution / DT KNX Series / Presence and Motion Detector

DT-KNX-360i-10-SENSO ao

All in One Sensor (presence detector) for ceiling mounting
Control system: KNX Bus-System
Technology:

- Passive infrared with Quad PIR and acoustic sensor

- Measurement of temperature, relative humidity and VOC/CO² equivalents

Safety:

- 24-hour production “Burn-in-Test” mode

Installation type:

Flush-mounted – Standard

- Surface-mounted – with DT-Surface Frame accessory

- Recessed ceiling installation – with DT-FM Set accessory

- Flush with ceiling – with DT-FM Set + DT-FM Flat Cover accessories

Installation position: Ceiling 
Recommended installation height: 2.80 m

Installation dimension: Ø 68 mm 
Weight: 62 g 
Operating voltage: 29-31 VDC (KNX)

Power consumption: 0.2 W

Detection angle: 360°

Range: Ø 10 m

Target brightness approx.: 0 - 2000 Lux [+ Daytime operation]

Switch-off time delay: up to 18 h

Operating type: Fully or semi-automatic

Functions:

- Extensive ETS parameterization options

- Nightlight

- Individual settings possible via IR remote control

- Remote control can be deactivated – tamper-proof

- Status display via RGB LED (can be deactivated)

- RGB LED light-dependent – adapts to ambient brightness

- Detection and microphone sensitivity adjustable

- Teach-in function – easy light value calibration

Interface: Push button input via KNX/ EIB push button

Connection: KNX Bus-System

Protection type: IP20

Protection class: III

Dimensions: Ø 84 mm, H: 30 mm

Area of application: Interior

Operating temperature: 0°C ... +50°C

Housing color: Anthracite [RAL 7021]

Lens color: Opal

Housing: UV-stabilized polycarbonate