Creative North Solution / DT 230V Series / Presence and Motion Detector

DT-230-360i-10-DALI-MIC wo

All in One Sensor (presence detector) for ceiling mounting
Control system: Constant light control as DALI broadcast
Technology: Passive infrared with Quad PIR and acoustic sensor
Safety:

- 24-hour production “Burn-in-Test” mode

- Integrated thermal protection against overload with LED indicator

Installation type:

Flush-mounted – Standard

- Surface-mounted – with DT-Surface Box w accessory

- Recessed ceiling installation – with DT-FM Set accessory

- Flush with ceiling – with DT-FM Set + DT-FM Flat Cover accessories

Installation position: Ceiling
Recommended installation height: 2.80 m

Installation dimension: Ø 68 mm
Weight: 62 g
Operating voltage: 230 VAC / 50-60 Hz

Power consumption: 0.2 W

Switching capacity: DALI interface 50mA for max. 25 DALI operating devices

Detection angle: 360°

Range: Ø 10 m

Target brightness approx.: 0 - 2000 Lux [+ Daytime operation]

Switch-off time delay: 1 sec. to 180 min.

Operating type: Fully or semi-automatic

Orientation light: Adjustable reduction of light output (10-50%)

Switch-off time delay orientation light: 1 min. to 60 min. / ON

Functions:

- Ready to use thanks to factory settings – Default

- 6 factory programs selectable by button

- Individual settings possible via IR remote control

- Motion or presence detector mode switchable via button

- Remote control can be deactivated – tamper-proof

- Status display via RGB LED (can be deactivated)

- RGB LED light-dependent – adapts to ambient brightness

- Detection and microphone sensitivity adjustable

- Teach-in function – easy light value calibration

Interface: Push button input

Connection:

- Plug-in connection cable – simple, fast, and cost-efficient

- Through-wiring with 3-way lever clamp (up to 2.5 mm²) – included

- Range extension – simple through master-master configuration

Protection type: IP20

Protection class: II

Dimensions: Ø 84 mm, H: 30 mm

Area of application: Interior

Operating temperature: 0°C ... +50°C

Housing color: White [RAL 9016]

Lens color: Opal

Housing: UV-stabilized polycarbonate